CMPS445- Embedded Systems

Team 2 Thursday

Team Members:

Mohamed Tarek Mohamed 1220195

Ali Khaled Hamdy 1220261

ElHussein Awad 1220226

Ibrahim Aly Ibrahim 1220159

Abdallah Mohamed ElMahdy 1220257

Youssef Salama 1210330

# 1. Mechanical Design

## Robot Dimensions

Length: 25 cm

Width: 15 cm

Height: 8 cm

Total Package: Fits within 25cm × 20cm constraint

## Wheel Configuration

Type: 4WD differential drive (skid steering)

Wheels: 4× 65mm pneumatic wheels

## Sensor Placement

Sensor Type: HC-SR04 Ultrasonic

Location: Front, Right and Left of robot

Orientation for side sensors: Perpendicular to wall (90° wall-normal)

Detection Range: 5-30 cm effective zone

## Sensor Detection Logic

The robot detects turns by monitoring distance rate of change:

Left Turn: Wall distance INCREASES (wall receding) → distance right < 15

Right Turn: Wall distance DECREASES (wall approaching) → distance left < 15cm

Straight: Distance stays constant → maintain 15 cm constant each side

# 2. Electrical Design

## Block Diagram

ESP32 at center

Motor control section (LEDC PWM for speed)

GPIO section (direction control pins)

Ultrasonic sensor section

H-Bridge drivers (L298N × 2)

4 DC Motors

Power distribution (12V battery)

## Pin Configuration

Motor Control Pins (PWM Speed):

GPIO12 → Left Front Motor speed (LEDC Channel 0)

GPIO13 → Left Rear Motor speed (LEDC Channel 1)

GPIO14 → Right Front Motor speed (LEDC Channel 2)

GPIO15 → Right Rear Motor speed (LEDC Channel 3)

Direction Control Pins:

GPIO16, GPIO17 → Left Front motor direction

GPIO18, GPIO19 → Left Rear motor direction

GPIO21, GPIO22 → Right Front motor direction

GPIO23, GPIO25 → Right Rear motor direction

Sensor Pins (Ultrasonic HC-SR04):

GPIO26 → TRIG (output, 3.3V pulse)

GPIO27 → ECHO (input, via voltage divider 5V→3.3V)

## Power Distribution

12V Battery supplies:

+12V → L298N H-Bridge 1 (left motors)

+12V → L298N H-Bridge 2 (right motors)

GND → Common ground (H-bridges, ESP32, sensors)

5V Regulator → HC-SR04 VCC (+5V)

# 3. FSM Design

## States

STRAIGHT_CORRIDOR - Following wall, maintaining distance

TURN_DETECTED - Wall distance changed significantly

EXECUTING_TURN - Rotating left or right (90°)

ERROR_WALL_HIT - Collision detected (d < 8cm)

ERROR_LOST_TRACK - Wall distance too large (d > 25cm)

ERROR_TURN_TIMEOUT - Turn taking too long (>5 sec)

COMPLETE - Track finished, send data

## Transition Table

| Current State | Trigger | Condition | Next State | Action |
| --- | --- | --- | --- | --- |
| STRAIGHT_CORRIDOR | Distance data | delta > 3cm | TURN_DETECTED | Classify LEFT |
| STRAIGHT_CORRIDOR | Distance data | delta < -3cm | TURN_DETECTED | Classify RIGHT |
| STRAIGHT_CORRIDOR | Distance data | d < 8cm | ERROR_WALL_HIT | Hard stop |
| STRAIGHT_CORRIDOR | Distance data | d > 25cm | ERROR_LOST_TRACK | Hard stop |
| TURN_DETECTED | Immediately | - | EXECUTING_TURN | Apply turn motors |
| EXECUTING_TURN | Time elapsed | ~1.5 sec | STRAIGHT_CORRIDOR | Resume forward |
| EXECUTING_TURN | Timeout | >5 sec | ERROR_TURN_TIMEOUT | Hard stop |
| ERROR_* | Immediately | - | COMPLETE | Stop, prep data |

## Motor Control During States

STRAIGHT_CORRIDOR:

If distance > 12cm (too far): left_speed = BASE + correction, right_speed = BASE - correction

If distance < 12cm (too close): left_speed = BASE - correction, right_speed = BASE + correction

If distance = 12cm (perfect): left_speed = BASE, right_speed = BASE

BASE = 180/255, correction = Kp × error, Kp = 15

EXECUTING_LEFT_TURN:

Left motors: 50/255 (slow)

Right motors: 200/255 (fast)

EXECUTING_RIGHT_TURN:

Left motors: 200/255 (fast)

Right motors: 50/255 (slow)

# 4. Algorithm Description

## Wall Following

Objective: Maintain constant distance (12 cm) from wall while moving forward

Method: Proportional Control

- Measure distance from ultrasonic sensor

- Compute error = |TARGET_DISTANCE - current_distance|

- Calculate correction = Kp × error (where Kp = 15)

- Adjust motor speeds to maintain target distance

## Turn Detection

Objective: Detect when robot reaches a 90° corner and classify direction

Method: Rate of Change Detection

- Compute: distance_delta = current_distance - previous_distance

- If |delta| < 3 cm: Stay in STRAIGHT_CORRIDOR

- If delta > 3 cm: LEFT TURN DETECTED (wall receding)

- If delta < -3 cm: RIGHT TURN DETECTED (wall approaching)

## Turn Counter

Counter incremented once per detected turn

Detection = significant distance change (±3 cm in one measurement)

No re-counting during rotation

Final format: "Turns: 5" + "Sequence: L, R, R, L, L"

## Key Thresholds

Target wall distance: 12 cm

Safe distance range: 8-25 cm

Turn detection threshold: ±3 cm

Turn execution time: ~1.5 seconds

Proportional gain (Kp): 15

# 5. Communication Plan

## Protocol: Bluetooth

Why WiFi:

Lower latency than Bluetooth

Sufficient bandwidth for turn data

Built into ESP32

Easy PC connection

## WiFi Configuration

SSID: RobotNet

Password: [To be determined]

Protocol: UDP (port 8888)

Fallback: TCP if UDP fails

Retry: 3 attempts, 2 sec timeout

## Data Format: JSON

{

"robot_id": "ROBOT_001",

"status": "complete",

"turn_count": 5,

"sequence": ["L", "R", "R", "L", "L"],

"time_ms": 15823,

"score_ms": 15823

}

## PC Receiver

Simple Python socket listener on port 8888

Receives JSON, logs to file, sends ACK

Can run on any PC connected to same WiFi network

# Summary

| Component | Specification |
| --- | --- |
| Microcontroller | ESP32 with WiFi |
| Sensor | HC-SR04 Ultrasonic |
| Motor Drivers | 2× L298N H-bridge |
| Motors | 4× DC motors |
| Drive Type | 4WD Differential (Skid Steering) |
| Target Wall Distance | 12 cm |
| Turn Detection Threshold | ±3 cm |
| Control Type | Proportional (Kp = 15) |
| Communication | WiFi JSON |
| Duty Cycle | 1 kHz PWM, 8-bit (0-255) |

Status: Phase 1 Design Complete - Ready for Implementation